## 🚌 Welcome to Your Elddis Sunseeker 115 GT C23 DON

Thank you for choosing to travel with our motorhome! This guide will help you get familiar with everything you need for a comfortable and stress-free trip.

### 🔑 Quick Start Checklist

Before setting off:

- ✅ Secure all windows, skylights, and cupboards and ensure all blinds are open when driving

- ✅ Close and lock the fridge

- ✅ Disconnect electric hookup

- ✅ Lower and secure TV aerial (if used)

- ✅ Ensure gas is turned off

- ✅ Check step is retracted

- ✅ ensure wastewater tap is closed

- ✅ Ensure clean water tank is closed

- ✅ Secure loose items

### 🚐 Driving & Dashboard Basics

- License Requirement: Standard UK driving license (Cat B)

- Fuel: Diesel – fill from the passenger side

- Handbrake: Pull up firmly to engage

- Gears: Manual – 5-speed gearbox

- Dimensions: ~5.7m long – check height/length signs when parking all dimensions are written on the underside of the driver sun visor

- Reversing: Use mirrors carefully; spotter recommended

### 🔌 Electric Hookup & Power

- Hookup Cable: Located in the rear outdoor storage box.

- Connection: Plug into campsite post first, then into van

- 12V System: Powers lights, water pump, TV Turn the master switch on which is located at the bottom of the TV cupboard when you enter the motorhome through the rear door. Also, the main panel on the side of the wardrobe the 12v switch should be on AUX never use the VEH.

- 240V System: Works when hooked up — the 2 sockets in the rear of the motorhome.

- Control Panel: Shows battery levels and water tank levels

- Solar system: this is setup, and you don’t need to do anything it will charge the leisure battery automatically when in direct sunlight.

### 🔥 Gas System

- Gas Bottle: Located in external locker

- To Use: Open the valve at the top of the bottle

- Gas Is Used For:

  - Cooker/hob

  - Fridge (if set to gas mode)

  - Heating (if not on electric hookup)

- Turn Off When Driving or when not in use

### 🚿 Water System

- Fresh Water Fill Point: External Fill point on driver side – use hose provided (it is recommended that you don’t drink the water) ensure that the water tap is closed its located halfway down the driver side of the van and is blue in colour. Also, ensure that the master drain valve for the water heater (located in the wardrobe under the lift out panel is flat this is yellow in colour. When draining lift the yellow switch upright to enable the boiler to drain

- Hot water: when you fill the water the hot water tank will automatically fill (providing the yellow switch mention above is flat). Once full of water before you turn on the ultra-store at the side of the wardrobe prime all the taps by turning on the hot water on each tap until you get clear running water. You can then turn on the ultra-store.

- Water Pump: Switch on at control panel on side of wardrobe

- Waste Water: Drains into grey water tank underneath then drive over designate wastewater grid on the campsite and empty. The tap is located at the rear of the vehicle just underneath the habitation door.

- Empty Waste: Use designated campsite dump stations

- Check how much fresh water you have onboard: press the water switch on the main control panel located on the side of the wardrobe. This will then give you a reading.

### ❄️ Fridge/Freezer

- Power Modes:

  - 12V (while driving) 12v does not work when parked up it is just for use when driving.

  - Gas (off-grid)

  - 240V (when hooked up)

- Switching Modes: Use fridge control panel to select power source

- Secure the Door: Use locking latch before driving

### 🍳 Cooking Appliances

- Gas Hob & Grill: Push knob in and turn while pressing igniter

- Ventilation: Open window or roof vent when cooking

### 🌡️ Heating & Hot Water

- Truma System:

  - Choose gas or electric mode if using gas mode for the heating or hot water please ensure you remove the cowl cover located on the driver’s side of the motorhome

  - Turn dial to desired temperature

  - Allow 30 mins to heat water

- Blown Air Heating: Warms up the interior – check vents are clear

### 🛏️ Sleeping Setup

- Side Lounge: Converts into a double bed

  - Slide slats out and rearrange cushions:

  - For bedding & pillows these are located under the bench seats

### 🚽 Toilet & Bathroom

- Cassette Toilet:

  - Use flush

  - If waste tank is full Indicator light will show on the main panel located on the side of the wardrobe.

- Empty Cassette:

  - Access from outside locker

  - Take to designated waste disposal area

  - Rinse and refill with toilet fluid

### 📺 TV & Entertainment

- TV/DVD Combo: Works on 12V

- Aerial: Manual or fixed – adjust for best signal

- 12V Socket: Available near TV point and on the dashboard (engine needs to be running to use)

### 🧼 Cleaning & Rubbish

- Cleaning Kit: Under sink or in rear locker

- Brush/Dustpan: In rear cupboard under TV

- Bins: Empty rubbish into the designated bins on the campsite

### 🧯 Safety Equipment

- Smoke alarm ✔️

- CO detector ✔️

- Fire extinguisher ✔️

- Fire blanket ✔️

### ❗ Emergency Info

- Breakdown Company: Call assist contact Number: 01603 216366 Give the registration and quote Policy Number MESZ06VR01 Call owner ASAP 07508043894

- Spare Bulbs/Fuses: stored in locker above travelling seats

- Spare wheel: located under the rear of the motorhome and the jack etc. is located under the bench seats in the rear of the motorhome.

- Further instructions are available in the folder in the cab overhead locker